proximity sensor parameters

| Model |  | PR08-2DN | PR12-4DN | PR18-8DN |  | PR30-15DN |
| --- | --- | --- | --- | --- | --- | --- |
| Sensing distance |  | 2mm | 4mm | 8mm |  | 15mm |
| Hysteresis |  | Max.10% of sensing distance |  |  |  |  |
| Standard sensing target |  | 8*8*1mm | 12*12*1mm (Iron) | 25*25*1mm (Iron) |  | 45*45*1mm (Iron) |
| Setting distance |  | 0 to 1.4mm | 0 to 2.8mm | 0 to 5.6mm |  | 0 to 10.5mm |
| Power supply (Operation voltage) |  | 12-24VDC (10-30VDC) |  |  |  |  |
| Current consumption |  | Max.10mA |  |  |  |  |
| Response frequency |  | 1kHz | 500Hz | 350Hz |  | 200Hz |
| Residual voltage |  | Max.2.0V | Max.1.5V |  |  |  |
| Affection by Temp |  | Max.±10% for sensing distance at ambient temperature 20℃, PROB Series:Max.±20% |  |  |  |  |
| Control output |  | Max.200mA |  |  |  |  |
| Isolation resistance |  | Min.50 MΩ(at 500VDC megger) |  |  |  |  |
| Dielectric strength |  | 1500VAC 50/60Hz for 1 minute |  |  |  |  |
| Vibration |  | 1mm amplitude at frequency of 10 to 55Hz(for 1 min) in each of X,Y,Z directions for 2 hours) |  |  |  |  |
| shock |  | 500m/s(approx,50G) in X, Y, Z directions for 3 times |  |  |  |  |
| Indicator |  | Operation indicator:Red LED |  |  |  |  |
| Environment | Ambient temperature | -25 to 70℃,storage:-30 to 80℃ |  |  |  |  |
|  | Ambient humidity | 30 to 95%RH,storage:35 to 95%RH |  |  |  |  |
| Protection circuit |  | Surge protection circuit, Reserve polarity protection circuit,Overcurrent protection circuit |  |  |  |  |
| Protection structure |  | Ip67(IEC standard) |  |  |  |  |
| Material |  | Case/Nut;Nickel plated Brass,Washer;Nickel plated Iron, Sensing surface:PBT, Standard cable(Black):Polyvinyl chloride(PVC), Oil resistant cable(Gray):Oil resistant Polyvinyl chloride(PVC) |  |  |  |  |
| Cable |  | Φ3.5mm,3-wire,2m9AwG24,Core diameter;0.08mm;Number of cores:40,Insulator diameter:1mm | Φ4mm, 3-wire,2m |  | Φ 5mm, 3-wire, 2m |  |
|  |  |  | AWG22,Core diameter:0.08mm,Number of cores:60, Insulator diameter:Φ1.25) |  |  |  |
| Approval |  | CE |  |  |  |  |
| Weight |  | PR:Approx 64g | PR:Approx 84g | PR:Approx122g |  | PRL:Approx207g |