| PHOTOELECTRIC SENSOR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Model |  |  |  | G24 |
| Outward appearance illustration |  |  |  |  |
| Overall dimension |  |  |  |  |
| Diffuse | Detection distance |  |  | 50cm |
|  | DC10-30V | NPN | NO | G24-3A50NA |
|  |  |  | NC | G24-3A50NB |
|  |  |  | NO+NC | G24-3A50NC |
|  |  | PNP | NO | G24-3A50PA |
|  |  |  | NC | G24-3A50PB |
|  |  |  | NO+NC | G24-3A50PC |
|  | AC90-250V | SCR Controllable silicon | NO | G24-2A50LA |
|  |  |  | NC | G24-2A50LB |
|  |  | Relay output |  | G24-2A50JC |
| Retroreflictive | Detection distance |  |  | 4m |
|  | DC10-30V | NPN | NO | G24-3B4NA |
|  |  |  | NC | G24-3B4NB |
|  |  |  | NO+NC | G24-3B4NC |
|  |  | PNP | NO | G24-3B4PA |
|  |  |  | NC | G24-3B4PB |
|  |  |  | NO+NC | G24-3B4PC |
|  | AC90-250V | SCR Controllable silicon | NO | G24-2B4LA |
|  |  |  | NC | G24-2B4LB |
|  |  | Relay output |  | G24-2B4JC |
| Trough beam | Detection distance |  |  | 5m |
|  | DC10-30V | NPN | NO | G24-3C5NA |
|  |  |  | NC | G24-3C5NB |
|  |  |  | NO+NC | G24-3C5NC |
|  |  | PNP | NO | G24-3C5PA |
|  |  |  | NC | G24-3C5PB |
|  |  |  | NO+NC | G24-3C5PC |
|  | AC90-250V | SCR Controllable silicon | NO | G24-3C5LA |
|  |  |  | NC | G24-2C5LB |
|  |  | Relay output |  | G24-3C5JC |
| Control output |  |  |  | DC:200mA |
| Consumption current |  |  |  | DC<15mA |
| Response time |  |  |  | DC<2ms |
| Direction angle |  |  |  | 3C to 10C |
| Detected object |  |  |  | Transparent of opaque body |
| Working environment temperature |  |  |  | below 25C to 55C |
| INtensity of illumination of working environment |  |  |  | Incandescent lamp under 3000LX |
| Shell material |  |  |  | Metal |
| Protection grade |  |  |  | IP54 |