| Detection method |  |  |  | Slot Type |
| --- | --- | --- | --- | --- |
| Model | DC-type | NPN | NO | E3S-GS15N |
|  |  |  | NC | E3S-GS15N2 |
|  |  |  | NO + NC |  |
|  |  | PNP | NO | E3S-GS15P |
|  |  |  | NC | E3S-GS15P2 |
|  |  |  | NO + NC |  |
| Detection range |  |  |  | 15mm |
| Detection range adjustment |  |  |  | Sensitivity adjuster |
| Target Detection |  |  |  | Opaque objects |
| Response time |  |  |  | 1ms |
| On delay |  |  |  | 1.5ms |
| Light source |  |  |  | Infrared light 660nm |
| Power supply voltage |  |  |  | DC Type : DC12-24V (6-36V) ripple (PP) 10% or less , AC type : AC110-220V (90-250V) 50 / 60Hz |
| Withstand voltage |  |  |  | Between AC1000V 50 / 60Hz 1min charging section and shell |
| Impact of voltage |  |  |  | Within the rated supply voltage range of 15% of rated power supply voltage value , 10% detection distance |
| Current consumption |  |  |  | N.P type : 20mA or less , A-type : 1.7mA or less |
| Control Output |  |  |  | NP type : 300mA or less , A-type : 400mA or less , J type : 2A below ( contact life : 100,000 times ) |
| Circuit Protection |  |  |  | NPD type : reverse connection protection , surge absorber , load short-circuit protection , A Type : Surge |
| Ambient temperature and humidity |  |  |  | When the action Storage: -30- + 65 ( with no icing or condensation) , When the action Storage: 35-95% RH |
| Effect of Temperature |  |  |  | Temperature range -30- + 65 , + 23 when , 15% within the detection distance Temperature range -25- + 60 , + 23 when , 10% within the detection distance |
| Insulation resistance |  |  |  | 50M or more rooms (DC500 megger ) charging part of the shell |
| Material |  |  |  | Shell : ABS detection surface (lens ): PMMA |
| Protection structure |  |  |  | IP67 (IEC standard ) |