Operating voltage is noted. 12-24VDC indicated on the label is actually 6-36VDC can be used (the package is available

Detection distance: 5mm ± 10%

Setting distance: 0-4mm
Hysteresis Distance: 10 percent below the detection distance
Sensing object: magnetic metal (non-magnetic metal detection distance decreases)
Standard sensing object: Iron 18 * 18 * 1mm
Frequency response: DC: 0.5kHz AC: 25Hz
Supply voltage: DC Type: DC12-24V (6-36V) Ripple (PP) 10% or less,
AC type: AC110-220V (36-250V) 50 / 60Hz
Withstand voltage: AC1000V 50 / 60Hz 1min charge between parts and enclosure