size. Installation mode; Nonshielded. model; DC type; NPN; NO; LJ8A3-2-Z/BX. NC; LJ8A3-2-Z/AX. NO+NC. PNP; NO; LJ8A3-2-Z/BY. NC; LJ8A3-2-Z/AY. NO+NC. second line—an advisory post; NO; LJ8A3-2-Z/EX. NC; LJ8A3-2-Z/DX. AC; second line—an advisory post; NO; LJ8A3-2-J/EZ. NC; LJ8A3-2-J/DZ. Three line; NO+NC. Detection distance; 2mm + 10%. Set the distance; 0 - 1.6mm. Lag distance; The detection range below 10%. Object detection; Magnetic metal (non magnetic metal detection distance decreases). Standard test object; Iron 12*12*1mm. Frequency response; DC:0.5kHz AC:25Hz. supply voltage; DC type: DC12 - 24V (6-36V) pulse (P-P) below 10%, AC: AC110 - 220V (36 - 250V) 50/60Hz. Withstanding voltage; AC1000V 50/60Hz 1min charging part and the shell.. The influence of voltage; Within the rated supply voltage range 15% rated supply voltage of + 10% detection distance. Current consumption; Type N.P: 13mA, D: 0.8mA, A: 1.7mA. Output control; Type N.P: 300mA, D: 200mA, A: 400mA. Loop protection; Type N.P.D: reverse connection protection, surge absorption, load short-circuit protection, type A: surge absorption. The environment temperature and humidity; When the movement, save the -30 +65 C (ice, frost), action, save time: 35 - 95%RH. The influence of temperature; The temperature range of -30 DEG C - +65, +23 C, + 15% detection distance The temperature range of -25 DEG C - +60, +23 C, + 10% detection distance. Insulation resistance; The above 50M (DC500 megameter) charging part and the shell.. texture of material; Shell: nickel plated brass surface detection: ABS. Protection structure; IP67 (IEC).